# ARM WindMaster+VectorNav Platform

# System Architecture

## Block Diagram

## System Initialization

After the STM32 Peripheral Boilerplate, the system does the following:

- Begins a 1 MHz timer (TIM2), the microsecond counter that drives system time.

- Starts up the RTC and SysTime module by initializing the global variables and linking to TIM2, enabling the 1Hz Pulse from the RTC, and setting the firmware time to the value stored in the RTC.

- Starts the Filesystem module by initializing buffers and setting the FATFS path.

- Starts the Tasker module by initializing an empty task queue.

- Starts the Calculator module by initializing buffers and the calculation state.

- Initializes the RS232 Transceivers by powering them on with GPIO pins.

- Initializes the UARTs by clearing all of the flags.

- Initializes the Windmaster sensor by setting up the DMA buffers and sending */r/n to put it in configuration mode.

- Initializes the Vectornav by setting up the DMA buffers and sending the stop-async command.

- Initializes the Telus module by setting up the DMA and clearing buffers.

- The system then polls the Vectornav for a GPS fix and syncs both the RTC and System times to GPS time.

- The System enters the main While-Loop.

## System Time

The internal system time relies on a 1 MHz microsecond counter (TIM2) which is corrected with the reliable 1 Hz Pulse-Per-Second output of the AB-RTCMC.

Each time the system receives a pulse from the AB-RTCMC, the PPS event interrupt is triggered:

- The TIM2 counter is sampled.

- The number of ticks since the last PPS event is calculated.

- The ticks-per-second estimate EMA is updated. (initial/nominal = 1,000,000)

  - Used to interpolate a millisecond-resolution timestamp.

- The global epoch seconds counter is incremented

- If a systime update is enqueued, the system epoch seconds are updated to the new time.

- The PPS event moving-average calculation is the only work that’s done in the ISR.

## The Main While(1) Loop

The main while-loop is a tight sequence of services which exit early when not enabled.

### The Recorder

## The Tasker Module

### Task Requirements

#### Task Scheduler

#### Task Handler

## The Command Shell Task

## The Calculator